NMDOT CAD → GIS Conversion Tool (V1)

User Guide

Version: 1.0
Prepared by: Narendra Lingutla
Organization: SRIRAMA, LLC / NMDOT
Platform: ArcGIS Pro 3.x+

1. Purpose

The NMDOT CAD → GIS Conversion Tool (V1) provides a standardized, no-code workflow to convert roadway design CAD files (DWG, DGN, DXF) into GIS-ready datasets using ArcGIS Pro.

The tool is designed for GIS analysts, planners, and engineers and does not require Python knowledge.

Primary goals:

- Consistent CAD → GIS conversion

- Correct coordinate system definition

- Repeatable, auditable outputs

- Minimal manual steps

2. What This Tool Does (V1 Scope)

This tool:

- Converts CAD files using ArcGIS Pro’s CAD To Geodatabase engine

- Creates a new output File Geodatabase per run

- Creates a staging Feature Dataset (CAD_RAW) per run

- Defines (does not project) the CAD coordinate system

- Optionally adds results directly to the active ArcGIS Pro map

- Supports recursive folder search for CAD files

3. What This Tool Does NOT Do (By Design)

The following are out of scope for V1 and will be addressed in future versions:

- Attribute remapping to HPMS or MIRE

- CAD standards validation

- Geometry correction or cleanup

- Coordinate system auto-detection

- LRS event table creation

- Quality control reporting

4. Important GIS Rule (Read Carefully)

CAD data is assumed to already be in a coordinate system.

This tool DOES NOT reproject CAD data.

It only defines the coordinate system provided by the user.

If the wrong .prj file is supplied, the output will be misaligned.

Always confirm the CAD coordinate system in:

- Autodesk Civil 3D

- Bentley OpenRoads

before running this tool.

5. Inputs Required

5.1 Input CAD Folder

- Folder containing CAD files (.dwg, .dgn, .dxf)

- Can optionally include subfolders

5.2 Output Folder

- Location where the output geodatabase will be created

- A new .gdb is created per run

5.3 Projection File (.prj)

- Required for correct spatial alignment

- Must match the coordinate system used in the CAD files

- Example:
NAD 1983 StatePlane New Mexico West FIPS 3003 (US Feet)

5.4 Project Name (Optional)

- Defaults to the CAD folder name

- Used for output naming only

5.5 Search Subfolders (Optional)

- Enable if CAD files are stored inside subdirectories

5.6 Auto-Add to Map (Optional)

- If enabled, results are added to the active ArcGIS Pro map

6. Output Structure

For each run, the tool creates:

<Project>_Converted_<timestamp>.gdb

│

└── CAD_RAW_<Project>_<timestamp>

├── Point

├── Polyline

├── Polygon

├── Annotation

├── TextPoint

└── Alignment (if present in CAD)

This structure ensures:

- No feature name collisions

- Full auditability

- Safe re-runs without overwriting previous results

7. Step-by-Step Usage

- Open ArcGIS Pro

- Open or create a Map

- Open the NMDOT CAD → GIS Toolbox

- Run CADToGDBWrapper

- Provide required inputs

- Click Run

- Review messages and map output

8. Common Issues & Troubleshooting

Output appears far from expected location

- Wrong .prj file selected

- CAD drawing is not in the assumed coordinate system

No CAD files found

- Enable Search subfolders

- Verify file extensions

Data not added to map

- Ensure a map is open before running the tool

9. Recommended Best Practices

- Always confirm CAD coordinate system before running

- Keep one project per run

- Do not edit the staging dataset directly

- Treat CAD_RAW as read-only source data

10. Versioning & Future Enhancements

This release is V1 (Foundation).

Planned future enhancements include:

- HPMS/MIRE schema mapping

- CAD standards validation

- QC reporting

- LRS integration

- Digital delivery / BIM workflows

11. Support

For issues or enhancement requests, contact:

Suresh Parvatoja, PE, MBA
SRIRAMA, LLC